Section § 393.48: Brakes to be operative.

## Guidance Q&A

Question 2: If a truck or truck tractor manufactured prior to July 25, 1980, and having 3 or more axles, has inoperable brakes on the front axle or some of the brake components are missing, would the vehicle be in violation of § 393.48?

Guidance: Yes. § 393.48(a) requires that all brakes with which the vehicle is equipped must be operable at all times. Although § 393.42(b)(1) provides an exception to the requirement for brakes on all wheels for trucks and truck tractors with 3 or more axles and manufactured prior to July 25, 1980, the exception does not affect the applicability of § 393.48 for those cases in which the vehicle is equipped with inoperable front wheel brakes or only has certain portions of the front wheel brake system (e.g., shoes, linings, chambers, hoses) in place.